Input files for Bellhop.

Version IN

The input structure is an extension of the one adopted for the original Bellhop model.

Presently it consist of three files (upper case letters):

- BELL.ENV

- LAYER.ENV

- BLOSS.ENV

Files a) and b) must always be present. File c) is required for user specified bottom loss table.

a)  File BELL.ENV

## (  1)	UPSLOPE CASE				!TITLE

(  2)	3500,0						!FREQ (Hz)

(  3)	0							!Flag for printout  ( 0 no, 1 yes)

(  4)	'S0T'						!IO SS VA (Filter opt, Bottom index, Vol. Att.)

(  5)	0    2.00     	2260.00,			! NBB NPRF BD

0.00    	1526.00,			! Z(1) C(1),  m, m/s

25.00    	1527.00,			! Z(2) C(2),  m, m/s

50.00    	1528.00,

75.00   	1518.00

95.00   	1511.00

165.00   	1509.00

…

…

1300.00   	1525.40

1400.00   	1527.20

1500.00   	1528.90

1750.00   	1533.10

2000.00    1537.40

2260.00    1541.73				! Z(i) C(i),  m, m/s with Z(i) =  BD

(  6)	'1', 0.00							!BI,   DUMMY

(  7)	1  15.00/						!ND,   SD1(m)

1  100.00/					!NRD,  RD1(m)

201  0.0  22.00/					!NRR,  RRI(Km)

(  8)	'I'							!R or I

(  9)	401   -30.00    30.00/				!NBEAMS,  A1(deg),  A2(deg)

(10)	10.00   2300.00     23.00			!STEP(m),  ZBOX(m),  RBOX(Km)

15.00     	2260.00			!RIP(Km),  BIP(m) range and depth for 2nd prf

0.00     	1530.00			! Z(1) C(1),  m, m/s

25.00    	1529.00			! Z(2) C(2),  m, m/s

50.00   	1528.00

75.00   	1518.00

95.00   	1511.00

…

…

1750.00   	1533.10

2000.00     	1537.40

2260.00     	1541.73				! Z(i) C(i),  m, m/s with Z(i) =  BIP

(11)   and so on for other profiles if present.

DESCRIPTION of parameters for BELL.ENV:

(1)	Title

Syntax:

Title

Description:

TITLE:Title of run enclosed in single quotes

- FREQUENCY

Syntax:

FREQ

Description:

FREQ: Frequency in Hz

- FLAG for BELL.PRT

Syntax:

FLAG

Description:

FLAG  to generate BELL.PRT. This file is used to monitor parameters.

- OPTIONS

Syntax:

IO, SS, VA

Description:

IO: type of interpolation to be used for the profile.

"S" for cubic spline. In this program version this is the only available option.

The user may check in file PRF.PLT the results for the presence of fit problems caused by

the spline option.

Spline yields a poor fit to certain kinds of curves, e.g. curves with sharp bends.

SS: Type of top boundary condition

Values of Sea State from 0 to 9

VA: Volume attenuation option. The values are:

"T" Thorp attenuation formula.

"N" no volume attenuation.

- SOUND SPEED PROFILE

Syntax:

NBB		NPRF			BD

Z(1)		C(1)

………………

Z(i)		C(i)      with Z(i) = BD

Description:

NBB:	 Number of bottom bounces. It is effective only on raytracing operations.

NPRF:	 Number of sound speed profiles. In range independent case NPRF = 1.

BD: 	 Depth at bottom of sound speed profile

This value is used to detect the last point when reading in the profile which follows.

Z(i) :	Depth (m). for each point of the profile

C(i)	Sound speed (m/s) for each point of the profile

### Note. It is mandatory that

-  the first point must be at 0 meter depth (surface)

-  the last point must be at BD

- OPTIONS

Syntax:

BI	DUMMY

Description:

BI: Bottom province index ranging from 0 to 9.

Index 0 : 		bottom loss table provided by the user

Indexes 1 – 9 : 	MGS provinces (ref . 2)

(7) 	SOURCE/RECEIVER DEPTHS AND RANGES

Syntax:

NSD	SD1

NRD	RD1	RD2

NR	RR1	RR2

Description:

NSD:  Number of source depth. It MUST always be 1 (maintained for compatibility with

older versions)

SD1:   source depth (m)

NRD:  Number of receiver depths. Only two choices are available:

100 	for contour matrix calculation (MC case)

1 	for single TL calculation (ORC case)

RD1:   Min receiver depth (m) for case MC or receiver depth for case ORC

RD2:   Max receiver depth (m) for case MC, not required for case ORC

NR:     The number of receiver ranges.

RR1:   First receiver range (Km)

RR2:   Last receiver range (Km)

(8)	RUN TYPE

Syntax:

RI

Description:

'R': generates a file for raytrace plots (no Transmission Loss calculation is performed)

'I' : performs ray path computation and computes incoherent Transmission Loss.

- BEAM FAN

Syntax:

NBEAMS	A1	A2

Description:

NBEAMS: Number of angle/beams >1

A1: First angle

A2: Last angle

A1 < A2

- NUMERICAL INTEGRATOR INFORMATION

Syntax:

STEP		ZBOX		RBOX

Description:

STEP: The step size used to compute the ray path (m)

ZBOX: The maximum depth to compute ray path (m) (>BD)

RBOX: The maximum range to compute ray path (Km) (>RR2)

The required step size depends on many parameters. This include frequency, size of sampling steps in  Sound Speed Profile, characteristics of surface duct, if present), depth of receiver.

If you use STEP=0.0  program uses an internal computed step-size and tell you what it picked.

You can change step size and angle increment ( [A2 - A1)/NBEAMS]) until the results converge to your required  accuracy. To obtain a smooth ray tracing  you should use a step-size less than smallest distance between Sound Speed profile data points.

Rays are traced until they exit the box (ZBOX,RBOX).

(11)	OTHER PROFILES

Syntax:

RIP	BIP

Z(1)	C(1)

…………..

Z(i)	C(i)  until Z(i) = BIP

Description:

#### RIP: Range (Km) of  Jth  profile with J going from 1 to 51

BIP: Depth (m) of Jth  profile with J going from 1 to 51

Z(i): depth (m)

C(i) sound speed (m/s)

b)  File LAYER.ENV

# ( 1)	2	!NBP

0.0       2260.0	!RKM	DM

22.0       260.0	!RKM	DM

( 2)	2	!NBR

0.0       1.0	!RKM	BI

22.0     1.0	!RKM	BI

( 3)	2	!NSS

0.0       0.0	!RKM	SS

22.0     0.0	!RKM	SS

Description of LAYER.ENV

(1)	BOTTOM PROFILE

Syntax:

NBP

RKM(i)		DM(i)

Description:

NBP: Number of Bottom Profile points

RKM: range(Km.)

DM: depth(m.)

i = 1 , NBP

NBP > 1

(2)	BOTTOM REGIONS (MGS INDEX)

Syntax:

NBR

RKM(i)		BI(i)

Description:

NBR: Number of Bottom Regions associated with an MGS index.

RKM: range(Km.)

BI: bottom regions index

i = 1 , NBR

NBR > 1

(3)	SEA STATE REGIONS

Syntax:

NSS

RKM(i)		SS(i)

Description:

#### NSS: Number of region for Sea State definition

RKM: range(Km.)

SS: Sea State

i = 1 , NSS

NSS > 1

c)   File BLOSS.ENV

( 1)	0.0	!COA

( 2)	0.0      0.0	!AN	LOSS

91.0    0.0	!AN	LOSS

Structure description of BLOSS.ENV file:

- Cut Off angle

Syntax:

COA

Description:

Cut Off Angle (degrees)

- Bottom loss profile

Syntax:

AN(i),	LOSS(i)

Description:

AN: grazing angle (degrees)

LOSS	: loss (dB)

i = 1 , 91

At least two pairs should be present with AN(i) =1 and AN(last) =90°

OUTPUT FILES

The output structure consists of three files (upper case letters):

- BELL.PRT (generated when the flag at point (3) in file BELL.ENV is set to 1)

- PRF.PLT     (always generated)

- RAY.PLT   (always generated)

- TLOSS.LOS (generated for NRD = 1 at point (7) in file BELL.ENV )

- MAT.MAT   (generated for NRD = 100 at point (7) in file BELL.ENV )

- File BELL.PRT

Example of content:

Overall modified profile

1    0.000000E+00     1526.000000

2        5.000000     1526.200000

3       25.000000     1527.000000

4       40.000000     1527.600000

…

…

40     1885.000000     1535.422000

41     2000.000000     1537.400000

42     2120.000000     1539.398000

43     2130.000000     1539.565000

44     2140.000000     1539.732000

45     2260.000000     1541.730000

D_DUCT,C_DUCT,IMAX :       48.499990     1528.163000           6

CRT ANG AT FIRST PROFILE :        2.612994

Too close to crit_angle. Dont trace ang.   0.000000E+00

-30.00    -28.00    -26.00    -24.00    -22.00    -20.00    -18.00    -16.00

-14.00    -12.00    -10.00     -8.00     -6.00     -4.00     -2.00      2.00

4.00      6.00      8.00     10.00     12.00     14.00     16.00     18.00

20.00     22.00     24.00     26.00     28.00     30.00

DELTAS =        10.000000, DELTASD =        2.000000

SOURCE DEPTH AND C       15.000000     1526.574000

Thorp Value :   2.314184E-04 [dB/m]

LOSS PRINTED ON MAT.MAT FILE

Numbers of Receiver Depth =           1

Numebers of Ranges     =         201

b)   File PRF.PLT

Example of content:

.152600E+04    .152700E+04    .152800E+04    .151800E+04    .151100E+04

...

.152890E+04    .153310E+04    .153740E+04    .154173E+04

.000000E+00    .250000E+02    .500000E+02    .700000E+02    .950000E+02

...

.150000E+04    .175000E+04    .200000E+04    .226000E+04

Description of structure:

##### This file can be used to monitor the input profiles. Each input profile and the modified one (see point (4) in the description of BELL.ENV file) are saved.

Two blocks of NPT values are used to describe each profile.

The first block contains the Sound Speed values, the second block contains the depth values.

The data is written with FORTRAN format: (5(1X,E12.5)).

These two blocks are repeated for every intermediate evaluation profile.

c)  File RAY.PLT

Example of content:

.000000E+00    .865807E+01    .203489E+02    .320417E+02    .437363E+02

.554333E+02    .671331E+02    .788356E+02    .905395E+02    .102238E+03

...

.751914E+04    .753239E+04    .754565E+04    .755891E+04    .757217E+04

.758542E+04    .759868E+04    .761194E+04    .762520E+04

.500000E+01   -.500470E-05    .675044E+01    .134976E+02    .202415E+02

.269814E+02    .337164E+02    .404465E+02    .471743E+02    .539093E+02

...

.802723E+03    .805274E+03    .807823E+03    .810371E+03    .812918E+03

.815464E+03    .818008E+03    .820550E+03    .823092E+03

Description of structure:

Two blocks of NPT values are used to describe each ray path.

The first block contains the range values, the second one contains the depth values.

The data is written with FORTRAN format (6(1X,E12.5)).

These two blocks are repeated for every ray.

The NPT value varies according to the ray paths.

d)  File TLOSS.LOS

Example of content:

.10000E-02   .11000E+00   .22000E+00   .33000E+00   .44000E+00   .55000E+00

.66000E+00   .77000E+00   .88000E+00   .99000E+00   .11000E+01   .12100E+01

…

Range Blocks

…

.20460E+02   .20570E+02   .20680E+02   .20790E+02   .20900E+02   .21010E+02

.21120E+02   .21230E+02   .21340E+02   .21450E+02   .21560E+02   .21670E+02

.21780E+02   .21890E+02   .22000E+02

.20000E+03   .93911E+02   .42984E+02   .46886E+02   .49504E+02   .51432E+02

.52923E+02   .54064E+02   .54992E+02   .55828E+02   .56609E+02   .57366E+02

Loss Blocks

…

…

.80132E+02   .79347E+02   .78316E+02   .76754E+02   .77055E+02   .75758E+02

.74479E+02   .72291E+02   .72794E+02

Description of structure:

Two blocks of  201 values compose the TLOSS.LOS file.

The first block contains the range values, the second one contains the loss values.

The data is written with FORTRAN format (6(1X,E12.5)).

e)   File MAT.MAT

.10000E-02   .11000E+00   .22000E+00   .33000E+00   .44000E+00   .55000E+00

.66000E+00   .77000E+00   .88000E+00   .99000E+00   .11000E+01   .12100E+01

…

…

.80132E+02   .79347E+02   .78316E+02   .76754E+02   .77055E+02   .75758E+02

.74479E+02   .72291E+02   .72794E+02

Description of structure:

The file MAT.MAT  contains the depth-range loss matrix, which consist of 100*100 cells.

Each cell gives the loss at  coordinates:

[ i * RangeMax / 100 ( i = 1:100),  j * DepthMax / 100 (j = 1:100) ]

with index `i` running first.

The matrix is written to the file in free field format.