Bellhop:  a Gaussian beam propagation loss model modified for fully range dependent operation.

Introduction.

The original Bellhop version (ref. 1) allowed for “range independent” calculation only.

In order to be used in operative scenarios it has now been converted to be a “fully range dependent” model.

The modifications allow active sonar simulations at frequencies higher than 1000 Hz.

In accordance with intended use the model has been complemented with very simple models for bottom and surface loss and its use has been restricted to perform incoherent calculation only.

Sound speed field.

A maximum of 51 different Sound Speed profiles can be specified along the range axis resulting in a maximum of 50 regions.

In each region the sound speed field is considered constant and is represented by the sound speed profile specified at the initial range of each region.

The model requires at least one sound velocity profile at range 0 Km.

The sound speed profiles along the range directions are inputted in sequence, (file BELL.ENV), and are identified by the value of the range at the begin of interval.

For each sound speed profile a cubic spline interpolation is performed to describe the sound speed field in depth.

To avoid artifacts caused by insufficient sound speed sampling (ref. 2) this version of the model automatically introduces some auxiliary points, which are computed by linear interpolations at significant depths.

Bottom topography and properties

The water depth (bottom) along the acoustic axis (range) is given by NBP pairs of (range, depth) values, with NBP varying from 2 to 51. This will result in NBP-1 discrete range intervals each having constant gradient. The constant gradient for each interval is derived from the two (range,depth) limiting points.

These range intervals do not coincide either for number or values, if not in particular cases, with the sound speed regions.

In range independent cases, flat bottom, there are two range pairs with same bottom depth.

The depth value at range 0, or first interval range, is maintained in the BELL.ENV file in order to retain compatibility with older versions of Bellhop, which are "range independent”.

In Bellhop a bottom loss index, with index values ranging from 0 to 9, is used to identify 10 different bottom tables.

The 0 index refers to a user provided table, while the indexes from 1 to 9 refer to the MGS provinces (ref. 3).

Each table is composed by bottom loss value in dB versus grazing angle, from 0 to 90 degrees.

The bottom properties along the acoustic axis (range) are represented by NBR pairs of (range,index) values, with NBR varying from 2 to 51, in order to provide NBR-1 discrete range intervals each having constant bottom properties.

The model requires at least two pairs of range, provinces values. The reason for two values depends from the necessity to maintain compatibility with other models.

In order to maintain compatibility with older range independent versions of Bellhop, the index for the first interval must also be given in file BELL.ENV.

Surface properties.

The sea state conditions along the acoustic axis (range) are represented by NSS pairs of range, indexes values, with NSS varying from 2 to 51, in order to provide NSS-1 discrete range intervals each of those having constant sea state conditions.

The ten indexes, from 0 to 9, represent the ten sea state codes.

Each table is composed by surface loss value in dB versus grazing angle, from 0 to 90 degrees.

The model requires at least two pairs of range, indexes values. The reason for two values depends from the necessity to maintain compatibility with other models.

In order to maintain compatibility with older range independent versions of Bellhop, the index for the first interval must also be given in file BELL.ENV.

Others modifications

Some other minor modifications have been introduced in Bellhop which do not interfere with the structure and the principal algorithm and formulas of the model.

Some of those are originated because the necessity of translate and execute the model on Personal Computer while some other to avoid particular operations not needed in this version.

It is therefore not necessary to list and comment them.

************************************

References.

- Porter, M. - The KRAKEN normal mode program. SACLANTCEN memorandum serial number SM-245, September 1991.

- Weinberg, H. - A continuos gradient curve fitting technique for acoustic ray analysis. J. Acoustic Soc. Am. 41, 419-634 (1971)

- Podesszwa, M. - Eugene, Acoustic Provinces charts based on MGS bottom loss data. NUSC report 4045, 4 May 1971