# Hyperbolic Frequency Modulation (HFM) or

# Linear Period Modulation (LPM) Signal Definition

X(t) = sin( (2/b) ln( 1 + bt/P1 ) )    for   0 < t < T

b = (f1  -  f2)/(f1 f2T)

T = Pulse Length

f1 = Start frequency ( t = 0 )

f2 = Stop frequency ( t = T )

P1 = 1/ f1   = The period of  f1

Why is it LPM?

f(t) = (1/2)d /dt     where   = (2/b) ln( 1 + bt/P1 )

f(t) = (1/2)d /dt = ( P1 + bt )-1

P(t) = f(t)-1 = P1 + bt

Example Plot for

T = .01 s

f1 = 1000 Hz

f2 = 2500Hz